Progress 1:

Project Name: mSIS, MicroSunInteliSense.

Description: It is a low cost, Lo-Ra-enabled IoT based monitoring system for off-grid solar micro-grids in rural area.

Currently, the hardware prototype model and the software program is work-in-progress.

Conceptualization of our Business Model is ready, but we are heldup due to some hardware integration and program interface issues; we are looking into it

Progress 2:

The Fist Prototype of the whole system comprises of the following parts:

  - The Sensor Node

    - Built around Atmega 328 PU processor based Arduino Uno R3 SBC

    - BME280 (H,T,P) Sensor (I2C), AHT21B (H,T) Sensor (I2C), or BPE680 (H,T,P,VOC- I2C/SPI) Sensor (I2C)

    - LDR/ Photo Transistor based Light Intensity Sensor

    - Solar PV Panel (No Wind Generator, Wind Speed and Wind Direction)

    - Battery Charge Level Monitoring, Load Current Monitoring

    - RFM98 LoRa Transceiver Radio Module with LoRa Antenna

    - Acts as a MQTT Publisher to Send Sensor Data to Gateway Node

  - The Edge Computing Gateway Node

    - Built around Raspberry Pi 3b+/4B SBC, running 64 bit Raspberry Pi OS Bookworm Lite

    - RFM98 LoRa Transceiver Radio Module with LoRa Antenna

    - Runs MQTT Broker and MQTT Subscriber to Receive Sensor Data from the Sensor Node

    - Uses SQLite to store sensor data locally

    - Runs Flask Web Server to host a Dashboard for Remote Monitoring

    - Runs various data analytic scripts to feed to the dashboard (efficiency calculation, SoC estimation, etc.)

  - The Remote Monitoring Cloud Services

    - User Interface: Responsive web app (PWA) based Dashboard accessible through a variety of computing and mobile infrastructures: Desktop/ Laptop PCS, PDAs and a variety of Handheld Devices

    - Displays Operational Status, Alerts and Maintenance Information

    - Runs AI/ML (TensorFlow Lite) for simple ML anomaly detection

Components Used

  - For the Sensor Node

    - Arduino Uno R3 SBC

    - AHT21B Sensor Module

    - Light Intensity Sensor

    - HopeRF RFM98 LoRa Transceiver Radio Module with LoRa PCB/ Rubber Duck Antenna

    - PV Panel

    - ACS712 Hall Effect Current Sensor

  - For the Gateway Node

    - Raspberry Pi 3b+

    - SSD1306 I2C 0.96" OLED Display

    - HopeRF RFM98 LoRa Transceiver Radio Module with LoRa PCB/ Rubber Duck Antenna